Printer start code

M201 X500.00 Y500.00 Z100.00 E5000.00 ;Setup machine max acceleration

M203 X500.00 Y500.00 Z10.00 E50.00 ;Setup machine max feedrate

M204 P500.00 R1000.00 T500.00 ;Setup Print/Retract/Travel acceleration

M205 X8.00 Y8.00 Z0.40 E5.00 ;Setup Jerk

M220 S100 ;Reset Feedrate

M221 S100 ;Reset Flowrate

M104 S{material_print_temperature} ;Start heating extruder

M140 S{material_bed_temperature} ;Start heating bed

M117 First Auto Home

G28 ; home all axes

M117 Auto Bed Level

G29 ; Auto Bed Level

M117 start heating

M190 S{material_bed_temperature} ;Wait for bed to reach temp before proceeding

M109 S{material_print_temperature} ;Wait for extruder to reach temp before proceeding

M83 ; extruder relative positioning

G90

G1 Y0 X0 Z15 F8000 ; move on cleaning position

T0 ; change tool

G92 E0 ; zero the extruded length

G1 Y1.2 X5 F1000 ; move on position

G1 Z{layer_height} F8000 ; move fast on position

M117 Moving 80mm

G91 ; relative positioning

G1 E78 F5000 ; restore - 2mm

G1 E2 X+10 F500 ; restore the rest (2 mm)

M400

G90 ; absolute positioning

G92 E0

M83

G1 X160 E14 F1000 ; intro line first section to right

G1 Y2.4 F1000 ; move on position

G1 X15 E14 F1000 ; intro line first section to left

G1 X0 E-1 Z{layer_height} F500 ; intro line going back to zero

M117 Cooling -50mm

G91 ; relative positioning

G1 E-6 F5000 ; stage 1 of 3 stage retraction

G1 Z+2 F8000 ; move fast on position

G1 E4 F2500 ; stage 2 of 3 stage retraction, this keeps from producing "hair"

G1 E-48 F5000 ; stage 3 of retraction, now cooling

M400

M117 Cooling T0 filament

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

M117 Moving to -80mm

G1 E-30 F1000 ; parking filament

M400

G90 ; absolute positioning

;↓↓↓ Intro line T1 ↓↓↓

T1 ; change tool

G92 E0 ; zero the extruded length

G1 Y3.6 X5 F1000 ; move on position

G1 Z{layer_height} F8000 ; move fast on position

;↓↓↓ Load from parking position ↓↓↓

M117 Moving 80mm

G91 ; relative positioning

G1 E78 F5000 ; restore - 2mm

G1 E2 X+10 F500 ; restore the rest (2 mm)

G90 ; absolute positioning

;↑↑↑ Load from parking position ↑↑↑

G92 E0

M83

G1 X160 E14 F1000 ; intro line first section to right

G1 Y4.8 F1000 ; move on position

G1 X15 E14 F1000 ; intro line first section to left

G1 X0 E-1 Z{layer_height} F500 ; intro line going back to zero

M117 Moving to -50mm

G1 E-6 F2500 ; release some presure

G1 Z+2 F8000 ; move fast on position

G1 E4 F2500 ; restore with backup

G92 E0 ; zero the extruded length

G1 E-48 F5000 ; stage 3 of retraction, now cooling

M400

M117 Cooling T0 filament

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

M400

M117 Moving to -80mm

G1 E-30 F5000 ; parking filament

G90 ; absolute positioning

;↑↑↑ Cooling filament and saving filament to parking position ↑↑↑

G92 E0 ; zero the extruded length

;↑↑↑ Intro line T1 ↑↑↑

Printer end code

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

M83 ; extruder relative positioning

G91 ; relative positioning

;↓↓↓ Cooling filament and saving filament to parking position ↓↓↓

G91 ; relative positioning

M117 active extruder -50mm

G1 E-6 F5000 ; stage 1 of 3 stage retraction

G1 Z+2 F8000 ; move fast on position

G1 E4 F2500 ; stage 2 of 3 stage retraction, this keeps from producing "hair"

G1 E-48 F5000 ; stage 3 of retraction, now cooling

M117 Cooling

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

M117 Active extruder -80mm

G1 E-30 F5000 ; parking filament

G90 ; absolute positioning

;↑↑↑ Cooling filament and saving filament to parking position ↑↑↑

G92 E0 ; zero the extruded length

G90 ; absolute positioning

G1 X0 Y0 F8000 ; move X/Y to 0

;↓↓↓ YOUR CODE ↓↓↓

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

G1 Z10 ;Raise Z more

G90 ;Absolute positioning

G1 X0 Y{machine_depth} ;Present print

M106 S0 ;Turn-off fan

M104 S0 ;Turn-off hotend

M140 S0 ;Turn-off bed

M84 X Y E ;Disable all steppers but Z

Extruder 1 Start code

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;↑↑↑ YOUR CODE ↑↑↑

T0

G90 ; absolute positioning

M117 Move to wasteblock

G1 X5 Y120 F8000 ; move ON the waste block

G1 X5 Y140 F3000 ; move on position

G91 ; relative positioning

G1 Z-2 F3600 ; touch the waste block

;↓↓↓ Load from parking position ↓↓↓

G91 ; relative positioning

G1 E78 F5000 ; restore - 2mm

G1 E7 Y-40 F500 ; restore the rest (2 mm) + 5mm

G90 ; absolute positioning

;↑↑↑ Load from parking position ↑↑↑

M117 Print w T0

G92 E0 ; zero the extruded length

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

Extruder 1 End Code

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

M83 ; extruder relative positioning

G91 ; relative positioning

G1 E-2 X-1 Y-1 F2500 ; release some presure and move

G1 Z+2 E-4 F2500 ; release even more presure

G90 ; absolute positioning

G1 X5 Y120 F8000 ; move ON the waste block

G91 ; relative positioning

G1 Z-2 X+10 E6 F2500 ; restore filament

G1 Y+2 F2500 ; move on position

G1 X-10 F2500 ; move on position

G1 Y-2 F2500 ; move on position

;↓↓↓ Cooling filament and saving filament to parking position ↓↓↓

G91 ; relative positioning

M117 retract to -50mm

G1 E-6 F5000 ; stage 1 of 3 stage retraction

G1 Z+2 F8000 ; move fast on position

G1 E4 F2500 ; stage 2 of 3 stage retraction, this keeps from producing "hair"

G1 E-48 F5000 ; stage 3 of retraction, now cooling

M117 Cooling T1

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

M117 move to -80

G1 E-30 F5000 ; parking filament

G90 ; absolute positioning

;↑↑↑ Cooling filament and saving filament to parking position ↑↑↑

G92 E0 ; zero the extruded length

G90 ; absolute positioning

;↓↓↓ YOUR CODE ↓↓↓

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

Extruder 2 Start Code

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;↑↑↑ YOUR CODE ↑↑↑

T1

G90 ; absolute positioning

M117 Move to wasteblock

G1 X5 Y120 F8000 ; move ON the waste block

G1 X5 Y140 F3000 ; move on position

G91 ; relative positioning

G1 Z-2 F3600 ; touch the waste block

;↓↓↓ Load from parking position ↓↓↓

G91 ; relative positioning

G1 E78 F5000 ; restore - 2mm

G1 E7 Y-40 F500 ; restore the rest (2 mm) + 5mm

G90 ; absolute positioning

;↑↑↑ Load from parking position ↑↑↑

M117 Print w T1

G92 E0 ; zero the extruded length

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

Extruder 2 End Code

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

M83 ; extruder relative positioning

G91 ; relative positioning

G1 E-2 X-1 Y-1 F2500 ; release some presure and move

G1 Z+2 E-4 F2500 ; release even more presure

G90 ; absolute positioning

G1 X10 Y120 F8000 ; move ON the waste block

G91 ; relative positioning

G1 Z-2 X+10 E6 F2500 ; restore filament

G1 Y+2 F2500 ; move on position

G1 X-10 F2500 ; move on position

G1 Y-2 F2500 ; move on position

;↓↓↓ Cooling filament and saving filament to parking position ↓↓↓

G91 ; relative positioning

M117 retract to -50mm

G1 E-6 F5000 ; stage 1 of 3 stage retraction

G1 Z+2 F8000 ; move fast on position

G1 E4 F2500 ; stage 2 of 3 stage retraction, this keeps from producing "hair"

G1 E-48 F5000 ; stage 3 of retraction, now cooling

M117 Cooling T1

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

G1 X+20 E7.5 F1000 ; move on position

G1 X-20 E-7.5 F1000 ; move on position

M117 move to -80

G1 E-30 F5000 ; parking filament

G90 ; absolute positioning

;↑↑↑ Cooling filament and saving filament to parking position ↑↑↑

G92 E0 ; zero the extruded length

G90 ; absolute positioning

;↓↓↓ YOUR CODE ↓↓↓

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;